catalogue

Chapter1 Product Introduction	4

1.1 Factory default	4

1.2 Get device information	4

Chapter 2 Wireless settings	5

2.1 Pairing setup code	5

2.2  BLE BLUETOOTH MODE	5

2.3 Mode Selection	5

2.4 Set the upload data speed delay	6

2.5 Check software version	6

2.6 Set sleep time	6

2.7 USB KBW	7

2.8 USB COM 	7

2.9 Output format	7

2.10 Output forced letter case conversion	9

Chapter 3 Input/Output Settings	10

Introduction	10

Startup Beeper	10

Trigger Click Beeper	10

Good Read and Error Read Indicators	11

Manual mode	12

Induction mode	13

Continuous mode	14

Chapter 4 Data Editing	18

Introduction	18

Add Preﬁx or Suﬃx	19

Example	20

Keyboard Operation	22

Clear Preﬁxes or Suﬃxes	23

Preﬁx Selections	23

Suﬃx Selections	24

Function Code Transmit	25

Chapter 5 Symbologies	26

Introduction	26

All Symbologies	27

1D Barcode	28

2D Barcode	28

Codabar	29

Code 39	33

Code 32 Pharmaceutical (PARAF)	36

Interleaved 2 of 5	38

NEC 2 of 5	41

Code 93	44

Straight 2 of 5 Industrial (three-bar start/stop)	45

Code 11	49

Code 128	51

ISBT 128 Concatenation	51

GS1-128	53

UPC-A	55

UPC-E0	58

UPC-E1	62

ISBN Translate	64

EAN/JAN-8	65

MSI	68

Check Characte	69

GS1 DataBar Omnidirectional	70

GS1 DataBar Limited	70

Korea Post	77

Check Digit	78

Han Xin Code	78

Maxi code	80

Micropdf	81

Composites	81

Codablock A	83

Scan the barcode to disable GS1 Data Matrix	85

Show Software Revision	86

2D Symbologies	91

ASCII Conversion Chart	93

Programming Charts	100

- 1、Product Introduction

  - Factory default

- End：Enter(\r)。

Factory default

  - Get device information

- Obtain the version number

2 Wireless settings

2.1 Pairing setup code

- ①Pair the receiver: Scan the following two barcodes in sequence.And connect the receiver to computer；

2.4G MODE                                       Connecting devices

②Pair Bluetooth devices in HID mode: Scan the following two barcodes in sequence, turn on the Bluetooth device to search for RB_Scanner_HID connection.

HID MODE                                                Connect

2.2  BLE BLUETOOTH MODE

Pairing Bluetooth devices: Scan the following two barcodes in sequence, and perform Bluetooth search RB_Scanner_BLE on the software that supports BLE

Connect.Note: BLE mode needs to be output on specific software (Bluetooth serial port)

BLE MODE                                                     Connect

2.3 Mode Selection

Instant upload mode                                Inventory Mode

Over-the-distance storage mode

①Operations in Inventory Mode

Upload all data                            Upload new data (that is, data that has not been uploaded before)

Display saved data                               Showing unuploaded data

Clear all data

2.4 Set the upload data speed delay

No delay                                        Delay 10ms

Delay 20ms

2.5 Check software version

Check the scanner software version             Check the receiving end version number

2.6 Set sleep time

X=1yyy(x=1000 means no sleep, sleep time calculation formula: yyy*10=z seconds)

No sleep                                         60 seconds

120 seconds                                       5 minutes

- 10 minutes

2.7 USB KBW

When the scanner is connected to the host using a USB receiver, the scanner can be configured as a standard keyboard input by scanning the USB KBW setup code.

USB KBW keyboard

2.8 USB COM

When the scanner is connected to the host using a USB receiver, the receiver virtual serial port output mode can be changed by scanning the USB COM setup code.

USB COM

2.9 Output format

UTF-8                                                      GBK

When the scanner recognizes it as a keyboard input device, different countries have different input characters, so you need to set different national languages. The keyboard defaults to the US language.

2.10 multi-language keyboards

U.S. (default)            Belgium (French)

Brazil (Portuguese)    Canada

Czech Republic         Denmark

Finland                     France

Germany                  Italy

Portugal                   Spain

Türkiye-F                 Türkiye-Q

U.K.                         Japan

Russia                    Vietnamese

2.11 Output forced letter case conversion

Letter conversion, when outputting barcodes with letter content, you can configure the output result to be all uppercase or all lowercase. For example, if the barcode content is: ab123dE, if you scan the "Convert to uppercase" barcode, the output result is: AB123DE; if you scan the "Convert to lowercase" barcode, the output result is: abc123de; the default case is not converted.

Case conversion

Do not convert                               Uppercase

Lowercase                          Case conversion

Chapter 3 Input/Output Settings

Introduction

This chapter mainly introduces the conﬁguration of the beep and LED of the barcode scanner when it is powered on, decoded, and triggered by the button.

Startup Beeper

The scanner can be programmed to beep when it’s started up. Default = Startup Beeper On.

(8410130.)

Startup Beeper Off

（8410131.)

* Startup Beeper On

Trigger Click Beeper

To hear an audible click every time the scanner button is pressed, scan the Trigger Click Beeper On barcode below. Default = Trigger Click Beeper Off.

(8410140.)

*Trigger Click Beeper Off

(8410141.)

Trigger Click Beeper On

Good Read and Error Read Indicators

Good Read Beeper

The beeper may be programmed On or Off in response to a good read. Default = Good Read Beeper On.

(8410010.)

Good Read Beeper Off

Good Read Beeper Volume

(8410011.)

* Good Read Beeper On

The beeper volume codes modify the volume of the beep the scanner emits on a good read. Default = High.

(8410091.)

Low

(8410092.)

Medium

(8410093.)

* High

(8410090.)

Off

Good Read Beeper Frequency

The beeper frequency codes modify the frequency of the beep the scanner emits on a good read. Default = Medium.

(8410061600.)

Low (1600 Hz)

(8410062400.)

* Medium (2400 Hz)

(8410064200.)

High (4200 Hz)

Good Read Beeper Duration

The beeper duration codes modify the length of the beep the scanner emits on a good read. Default = Normal .

(8410020.)

* Normal

Error Read Beeper Frequency

(8410021.)

Short

The beeper frequency codes modify the frequency of the sound the scan- ner emits when there is a bad read or error. Default = Razz.

(841007250.)

* Razz (250 Hz)

(8410073250.

Medium (3250 Hz)

(8410074200.)

High (4200 Hz)

Good Read LED

The LED indicator can be programmed On or Off in response to a good read. Default = On.

(8410081.)

* Good Read LED On

(8410080.)

Good Read LED Off

Manual mode

The manual reading mode is the default reading mode. In this mode, the recognition engine starts reading the code after the user presses the trigger key, and stops reading the code after successfully outputting information or the user releases the trigger key.

(8610020.)

Induction mode

After setting up, there is no need to trigger, and the recognition engine immediately starts monitoring the brightness of the surrounding environment. When the scene changes, the recognition engine waits for the set stabilization time to end before starting to read the code. If the following situation does not occur, the reading engine will cycle in the above way: if the barcode is not scanned within a single reading time, the reading engine will automatically pause reading and enter the monitoring state. In the induction reading mode, the reading engine can also start reading the code after the user presses the trigger button. When the code reading is successful and outputs information or the user releases the trigger button, it will switch to induction mode for 5 seconds and continue to monitor the brightness of the surrounding environment.

(8610023.)

sensitivity

Self sensing sensitivity, the higher the sensitivity, the easier it is to wake up, but it is easy to accidentally wake up the device to decoding mode, default to high.

High(8610054.)*

Mid(8610055.)

Low(8610056.)

Continuous mode

After setting up, there is no need to trigger. The reading engine immediately starts reading the code. During the reading process, the user can also manually pause the reading by clicking the trigger button. When the reading is successful and outputs information or the user releases the trigger button, it will switch to continuous mode for 5 seconds

(8610028.)

Same barcode reading delay

Sensing mode and continuous mode scanning can be used to set the time interval between barcode scanning equipment reading the next barcode after reading one barcode.。

(8510060.)

No time interval

(851006500.)

(500ms)

(851006750.)

(750 ms)

(8510061000.)

(1,000 ms)

(8510061500.)

(1,500 ms

Chapter 4 Data Editing

Introduction

This chapter describes how to add preﬁxes and suﬃxes.

- Default preﬁx = None. Default suﬃx = None.

- A preﬁx or suﬃx may be added or cleared from one symbology or all symbologies.

- You can add any preﬁx or suﬃx from the ASCII Conversion Chart deplus Code I.D. and AIM I.D.

- Enter preﬁxes and suﬃxes in the order in which you want them to appear on the output.

- When setting up for speciﬁc symbologies (as opposed to all symbologies), the speciﬁc symbology ID value counts as an added preﬁx or suﬃx character.

- The maximum size of a preﬁx or suﬃx conﬁguration is 200 characters, which includes header information.

Add Preﬁx or Suﬃx

Step 1. Scan the Add Preﬁx or Add Suﬃx symbol

Step 2. Determine the 2 digit Hex value from the Symbology Chart for the symbology to which you want to apply the preﬁx or suﬃx. For example, for Code 11, Code ID is “h” and Hex ID is “68”.

Step 3. Scan the 2 hex digits from the Programming Chart inside the back cover of this manual or scan 9, 9 for all symbologies.

Step 4. Determine the hex value from the ASCII Conversion Chart , for the preﬁx or suﬃx you wish to enter.

Step 5. Scan the 2 digit hex value from the Programming Chart inside the back cover of this manual.

Step 6. Repeat Steps 4 and 5 for every preﬁx or suﬃx character.

Step 7. To add the Code I.D., scan 5, C, 8, 0.

To add AIM I.D., scan 5, C, 8, 1.

To add a backslash (\), scan 5, C, 5, C.

Step 8. Scan Save to exit and save, or scan Discard to exit without saving.

(889002.)

Add Preﬁx

(888002.)

Add Suﬃx

(800002.)

Save

(800000.)

Disgard

Example

Add a Suﬃx to a speciﬁc symbology

To send a CR (carriage return)Suﬃx for code 128. only:

Step 1. Scan Add Suﬃx.

Step 2. Determine the 2 digit hex value from the Symbology Charts for code 128.

Step 3. Scan 6, 3 from the Programming Chart inside the back cover of this manual.

Step 4. Determine the hex value from the ASCII Conversion Chart , for the CR (carriage return).

Step 5. Scan 0, D from the Programming Chart inside the back cover of this manual.

Step 6. Scan Save, or scan Discard to exit without saving.

(888002.)

Add Suﬃx

(KAK.)

A

(SDS.)

D

(K6K.)

6

(K0K.)

0

(800002.)

Save

To Add a Carriage Return Suﬃx to All Symbologies

Scan the following barcode if you wish to add a carriage return suﬃx to all symbologies at once. This action ﬁrst clears all current suﬃxes, then programs a carriage return suﬃx for all symbologies.

(890000.)

Add CR Suﬃx All Symbologies

To Add a Line Break Suﬃx to All Symbologies

Scan the following barcode if you wish to add a line break suﬃx to all symbologies at once. This action ﬁrst clears all current suﬃxes, then programs a line break suﬃx for all symbologies.

(888002990A.)

Add LF Suﬃx All Symbologies

To Add a Carriage Return &a Line Break Suﬃx to All Symbologies

Scan the following barcode if you wish to add a carriage return suﬃx and a line break suﬃx to all symbologies at once. This action ﬁrst clears all current suﬃxes, then programs a carriage return suﬃx and a line break suﬃx for all symbologies.

(888002990D0A.)

Add CR and LF Suﬃx All Symbologies

Keyboard Operation

Different operations can be performed on the keyboard through conﬁguration during decoding output, such as automatic saving after decoding output.

Step 1: determine the hexadecimal value corresponding to the keyboard operation to be performed from the ASCII conversion of keyboard operation, and Determine the 2-digit hexadecimal value of the barcode to be set

Step 2. scan the barcode of "add keyboard operation".

Step 3. Determine the sequence of keyboard operation and barcode output. If keyboard operation is in front, scan "add preﬁx" barcode, and then scan "add suﬃx" barcode.

Step 4. Scan the corresponding 4-digit hexadecimal values in the Programming Charts of this manual according to the corresponding hexadecimal values (including barcode type and corresponding keyboard operation)

Step 5. Scan "save" barcode.

Step 6, scan "end adding keyboard operation"

（8210042）

Add keyboard operation

（8210040）

End adding keyboard operation

Example: add operation that automatic preservation after decoding output for all kinds of barcodes

First, conﬁrm the operation to be performed: save after barcode output, so suﬃx should be added after output barcode. Then determine the corresponding hexadecimal value according to the table in the appendix,all kinds of barcodes correspond to “9”“9”，The save operation corresponds to “1”“3”。

After conﬁrmation, scan "add keyboard operation" barcode, add suﬃx barcode, 9, 9, 1, 3, and then scan "save" barcode and “end adding keyboard operation” barcode

(Here "9" and "9" correspond to all coding systems, and "1" and "3" correspond to decoding output and saving)

Clear Preﬁxes or Suﬃxes

You can clear a single preﬁx or suﬃx, or clear all preﬁxes/suﬃxes for a symbology. If you have been entering preﬁxes and suﬃxes for single sym- bologies, you can use Clear One Preﬁx (Suﬃx) to delete a speciﬁc char- acter from a symbology. When you Clear All Preﬁxes (Suﬃxes), all the preﬁxes or suﬃxes for a symbology are deleted.

Step 1. Scan the Clear One Preﬁx or Clear One Suﬃx symbol.

Step 2. Determine the 2 digit Hex value from the Symbology Charts for the symbology from which you want to clear the preﬁx or suﬃx.

Step 3. Scan the 2 digit hex value from the Programming Chart inside the

back cover of this manual or scan 9, 9 for all symbologies.

Step 4. Scan the Save symbol.

(889004.)

Clear One Preﬁx

(888004.)

Clear One Suﬃx

(800002.)

Save

Preﬁx Selections

(889002.)

Add Preﬁx

(889004.)

Clear One Preﬁx

(889003.)

Clear All Preﬁxes

Suﬃx Selections

(888002.)

Add Suﬃx

(888003.)

Clear All Suﬃxes

(888004.)

Clear One Suﬃx

Function Code Transmit

When this selection is enabled and function codes are contained within the scanned data, the scanner transmits the function code to the terminal. Default = Disable.

(8080071.)

*Enable

(8080070.)

*Disable

Chapter 5 Symbologies

Introduction

Each type of barcode has its own unique properties. The barcode scanner can be adjusted to accommodate these property changes through the conﬁguration code in this chapter. The fewer the barcode types, the faster the barcode scanner can read. You can disable the barcode scanner to read the barcode types that will not be used to improve the performance of the barcode scanner.

All Symbologies

If you want to decode all the symbologies allowable for your scanner, scan the All Symbologies On barcode. If on the other hand, you want to decode only a particular symbology, scan All Symbologies Off followed by the On barcode for that particular symbology.

(9990011.)

All Symbologies On

(9990010.)

All Symbologies Off

Note: When All Symbologies On is scanned, 2D Postal Codes are not enabled.

2D Postal Codes must be enabled separately.

Message Length Description

You are able to set the valid reading length of some of the barcode symbolo- gies. If the data length of the scanned barcode doesn’t match the valid read- ing length, the scanner will issue an error tone. You may wish to set the same value for minimum and maximum length to force the scanner to read ﬁxed length barcode data. This helps reduce the chances of a misread.

EXAMPLE: Decode only those barcodes with a count of 6-10 characters.

Min. length = 06 Max. length = 10

Step 1. Select the barcode symbology to set the maximum reading length or the minimum reading length, scan the Minimum Message Length barcode in its catalog, and scan the number “6” and “Save” barcodes from the Programming Chart .

Step 2. Scan the Maxmum Message Length barcode and scan the numbers

1, 0 barcode and Save barcode from the Programming Chart . The above process sets the selected barcode symbology small reading length to 6 and the maximum reading length to 10

EXAMPLE: Decode only those barcodes with a count of 13 characters.

Min. length = 13 Max. length = 13

1D Barcode

If the bar code scanning device needs to decode all the one-dimensional code systems, please scan the bar code of “All 1D Barcode on”. Only solve speciﬁc code system, please scan " All 1D Barcode off" .

(9950040.)

All 1D Barcode on

2D Barcode

(9950041.)

All 1D Barcode off

If the bar code scanning device needs to decode all the two-dimensional code systems, please scan the bar code of “All 2D Barcode on”. Only solve speciﬁc code system, please scan " All 2D Barcode off"

(9950070.)

All 2D Barcode on

(9950071.)

All 2D Barcode on

Codabar

On/Off

(9000031.)

* On

Start/Stop Characters

(900000.)

Default All Codabar Settings

(9000030.)

Off

Start/Stop characters identify the leading and trailing ends of the barcode. You may either transmit, or not transmit Start/Stop characters.Default = Don’t Transmit.

(9000061.)

Transmit

Check Character

(9000060.)

* Don’t Transmit

No Check Character indicates that the scanner reads and transmits barcode data with or without a check character.

When Check Character is set to Validate and Transmit, the scanner will only read Codabar barcodes printed with a check character, and will transmit this character at the end of the scanned data.

When Check Character is set to Validate, but Don’t Transmit, the unit will only read Codabar barcodes printed with a check character, but will not transmit the check character with the scanned data. Default = No Check Character.

(9000010.)

* No Check Character

(9000012.)

Validate and Transmit

(9000011.)

Validate but Don’t Transmit

Concatenation

Codabar supports symbol concatenation. When you enable concatena- tion, the scanner looks for a Codabar symbol having a “D” start character, adjacent to a symbol having a “D” stop character. In this case the two mes- sages are concatenated into one with the “D” characters omitted.

A 1   2 3 4 D	D 5 6 7 8A

Select Require to prevent the scanner from decoding a single “D” Codabar symbol without its companion. This selection has no effect on Codabar symbols without Stop/Start D characters.

(9000021.)

On

(9000020.)

* Off

(9000022.)

Require

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 2-60. Minimum Default = 4, Maximum Default = 60.

(900005.)

Minimum Message Length

(900004.)

Maximum Message Length

Code 39

< Default All Code 39 Settings >

(901000.)

Default All Code 39 Settings

Code 39 On/Off

(9010011.)

* On

(9010010.)

Off

Start/ Stop Characters

Start/Stop characters identify the leading and trailing ends of the barcode. You may either transmit, or not transmit Start/Stop characters. Default = Don’t Transmit.

(9010091.)

Transmit

(9010090.)

* Don’t Transmit

Check Character

No Check Character indicates that the scanner reads and transmits bar- code data with or without a check character.

When Check Character is set to Validate, but Don’t Transmit, the unit only reads Code 39 barcodes printed with a check character, but will not transmit the check character with the scanned data.

When Check Character is set to Validate and Transmit, the scanner only reads Code 39 barcodes printed with a check character, and will transmit this character at the end of the scanned data. Default = No Check Character.

(9010040.)

* No Check Character

(9010041.)

Validate, but Don’t Transmit

(9010042.)

Validate and Transmit

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths

= 0-48. Minimum Default = 0, Maximum Default = 48.

(901008.)

Minimum Message Length

(901007.)

Maximum Message Length

Code 39 Append

This function allows the scanner to append the data from several Code 39 barcodes together before transmitting them to the host computer. When the scanner encounters a Code 39 barcode with the append trigger char- acter(s), it buffers Code 39 barcodes until it reads a Code 39 barcode that does not have the append trigger. The data is then transmitted in the order in which the barcodes were read (FIFO). Default = Off.

(9010021.)

On

Example

(9010020.)

* Off

After sanning on barcode, scan the three bar codes below in order. The barcode scanner does not output any data until the last bar code is scanned. After scanning the ESS barcode, the SUCCESS word is output correctly.

SU

CC

ESS

Code 32 Pharmaceutical (PARAF)

Code 32 Pharmaceutical is a form of the Code 39 symbology used by Ital- ian pharmacies. This symbology is also known as PARAF.

When you conﬁgure code32, you need to turn on code39 before you conﬁgure it.

(9010051.)

On

(9010050.)

* Off

FULL ASCII

If Full ASCII Code 39 decoding is enabled, certain character pairs within the bar- code symbol will be interpreted as a single character. For example:$V will be decoded as the ASCII character SYN, and /C will be decoded as the ASCII character #. Default = Off.

| NUL%U | DLE $P | SP SPACE | 0 | 0 | @%V | P | P | ‘ %W | p +P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SOH$A | DC1 $Q | ! /A | 1 | 1 | A A | Q | Q | a +A | q +Q |
| STX $B | DC2 $R | “ /B | 2 | 2 | B B | R | R | b +B | r +R |
| ETX $C | DC3 $S | # /C | 3 | 3 | C C | S | S | c +C | s +S |
| EOT $D | DC4 $T | $ /D | 4 | 4 | D D | T | T | d +D | t +T |
| ENQ $E | NAK $U | % /E | 5 | 5 | E E | U | U | e +E | u +U |
| ACK $F | SYN $V | & /F | 6 | 6 | F F | V | V | f +F | v +V |
| BEL $G | ETB $W | ‘ /G | 7 | 7 | G G | W | W | g +G | w +W |
| BS $H | CAN $X | ( /H | 8 | 8 | H H | X | X | h +H | x +X |
| HT $I | EM $Y | ) /I | 9 | 9 | I I | Y | Y | i +I | y +Y |
| LF $J | SUB $Z | * /J | : | /Z | J J | Z | Z | j +J | z +Z |
| VT $K | ESC %A | + /K | ; | %F | K K | [ | %K | k +K | { %P |
| FF $L | FS %B | , /L | < | %G | L L | \ | %L | l +L | ¦ %Q |
| CR $M | GS %C | - /M | = | %H | M M | ] | %M | m +M | } %R |
| SO $N | RS %D | . /N | > | %I | N N | ^ | %N | n +N | ~ %S |
| SI$O | US %E | / /O | ? | %J | O O | _ | %O | o +O | DEL %T |

Character pairs /M and /N decode as a minus sign and period respectively. Character pairs /P through /Y decode as 0 through 9.

(9010031.)

FULL  ASCII On

(9010030.)

*  FULL  ASCII Off

Interleaved 2 of 5

< Default All Interleaved 2 of 5 Settings >

On/Off

(902000.)

Default All Interleaved 2 of 5 Settings

(9020021.)

* On

Check Digit

(9020020.)

Off

No Check Digit indicates that the scanner reads and transmits barcode data with or without a check digit.

When Check Digit is set to Validate, but Don’t Transmit, the unit only reads Interleaved 2 of 5 barcodes printed with a check digit, but will not transmit the check digit with the scanned data.

When Check Digit is set to Validate and Transmit, the scanner only reads Interleaved 2 of 5 barcodes printed with a check digit, and will transmit this digit at the end of the scanned data. Default = No Check Digit.

(9020010.)

* No Check Digit

(9020012.)

Validate and Transmit

(9020011.)

Validate, but Don’t Transmit

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths

= 2-80. Minimum Default = 4, Maximum Default = 80.

(902004.)

Minimum Message Length

(902003.)

Maximum Message Length

NEC 2 of 5

On/Off

< Default All NEC 2 of 5 Settings >

(903000.)

Default All NEC 2 of 5 Settings

(9030011.)

* On

(9030010.)

Off

Check Digit

No Check Digit indicates that the scanner reads and transmits barcode data with or without a check digit.

When Check Digit is set to Validate, but Don’t Transmit, the unit only reads NEC 2 of 5 barcodes printed with a check digit, but will not transmit the check digit with the scanned data.

When Check Digit is set to Validate and Transmit, the scanner only reads NEC 2 of 5 barcodes printed with a check digit, and will transmit this digit at the end of the scanned data. Default = No Check Digit

(9030020.)

* No Check Digit

(9030022.)

Validate and Transmit

(9030021.)

Validate, but Don’t Transmit

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths

= 2-80. Minimum Default = 4, Maximum Default = 80.

(903004.)

Minimum Message Length

(903003.)

Maximum Message Length

Code 93

On/Off

< Default All Code 93 Settings >

(904000.)

Default All Code 93 Settings

(9040021.)

* On

(9040020.)

Off

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 0-80. Minimum Default = 0, Maximum Default = 80.

(904004.)

Minimum Message Length

(904003.)

Maximum Message Length

Straight 2 of 5 Industrial (three-bar start/stop)

<Default All Straight 2 of 5 Industrial Settings>

(905000.)

Default All Straight 2 of 5 Industrial (three-bar start/stop)Settings

On/Off

(9050011.)

On

(9050010.)

* Off

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-48. Minimum Default = 4, Maximum Default = 48.

(905003.)

Minimum Message Length

(905002.)

Maximum Message Length

(906000.)

Default All Straight 2 of 5 IATA (two-bar start/stop)Settings

On/Off

(9060011.)

On

Message Length

（9060010.）

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-48. Minimum Default = 4, Maximum Default = 48.

(906003.)

Minimum Message Length

(906002.)

Maximum Message Length

On/Off

(907000.)

Default All Matrix 2 of 5 Settings

(9070011.)

On

Message Length

(9070010.)

* Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-80. Minimum Default = 4, Maximum Default = 80.

(907003.)

Minimum Message Length

(907002.)

Maximum Message Length

Check

Scan the barcode below to enable or disable the check function of matrix25.

(9070051.)

Enable Check Function

(9070050.)

Disable Check Function

Code 11

On/Off

<Default All Settings>

(908000.)

Default All Code 11 Settings

(9080021.)

On

(9080020.)

* Off

Check Digits Required

This option sets whether 1 or 2 check digits are required with Code 11 barcodes. Default = Two Check Digits.

(3110280.)

One Check Digit

(3110281.)

* Two Check Digits

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-80. Minimum Default = 4, Maximum Default = 80.

(908004.)

Minimum Message Length

(908003.)

Maximum Message Length

Code 128

<Default All Code 128 Settings>

(909000.)

Default All Code 128 Settings

On/Off

(9090011.)

* On

(9090010.)

Off

ISBT 128 Concatenation

(9020051.)

ISBT 128 On

(9020050.)

*ISBT 128 Off

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 0-80. Minimum Default = 0, Maximum Default = 80.

(909003.)

Minimum Message Length

(909002.)

Maximum Message Length

GS1-128

<Default All GS1-128 Settings>

On/Off

(9100011.)

* On

Message Length

(910000.)

Default All GS1-128 Settings

(9100010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-80. Minimum Default = 1, Maximum Default = 80.

(910003.)

Minimum Message Length

(910002.)

Maximum Message Length

Telepen

<Default All Telepen Settings>

On/Off

(911000.)

Default All Telepen Settings

(9110011.)

On

Message Length

(9110010.)

* Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-60. Minimum Default = 1, Maximum Default = 60.

(911003.)

Minimum Message Length

(911002.)

Maximum Message Length

UPC-A

<Default All UPC-A Settings>

(912000.)

Default All UPC-A Settings

(9120031.)

* On

(9120030.)

Off

Check Digit

This selection allows you to specify whether the check digit should be transmitted at the end of the scanned data or not. Default = On.

(9120041.)

* On

(9120040.)

Off

Number System

The numeric system digit of a U.P.C. symbol is normally transmitted at the beginning of the scanned data, but the unit can be programmed so it will not transmit it. Default = On.

(9120051.)

* On

Addenda

(9120050.)

Off

This selection adds 2 or 5 digits to the end of all scanned UPC-A data. Default = Off for both 2 Digit and 5 Digit Addenda.

(9120011.)

2 Digit Addenda On

(9120010.)

* 2 Digit Addenda Off

(9120021.)

5 Digit Addenda On

(9120020.)

* 5 Digit Addenda Off

Addenda Required

When Required is scanned, the scanner will only read UPC-A barcodes that have addenda. You must then turn on a 2 or 5 digit addenda. Default = Not Required.

(9120061.)

Required

Addenda Separator

(9120060.)

* Not Required

When this feature is on, there is a space between the data from the barcode and the data from the addenda. When turned off, there is no space. Default

= On.

(9120071.)

* On

Note

(9120070.)

Off

Scan the barcode below to convert UPC-A to EAN_13 or not.

(9120111.)

Convert

(9120110.)

Not convert

UPC-E0

On/Off

<Default All UPC-E Settings>

(914000.)

Default All UPC-E0 Settings

Most U.P.C. barcodes lead with the 0 number system. To read these codes, use the *UPC-E0 On selection. If you need to read codes that lead with the 1 number system, use UPC-E1. Default = On.

(9140101.)

* UPC-E0 On

(9140100.)

UPC-E0 Off

Expand

UPC-E Expand expands the UPC-E code to the 12 digit, UPC-A format. Default = Off.

(9140021.)

On

(9140020.)

* Off

Addenda Required

When Required is scanned, the scanner will only read UPC-E barcodes that have addenda. Default = Not Required.

(9140031.)

Required

Addenda Separator

(9140030.)

* Not Required

When this feature is On, there is a space between the data from the barcode and the data from the addenda. When turned Off, there is no space. Default = On

(9140041.)

* On

Check Digit

(9140040.)

Off

Check Digit speciﬁes whether the check digit should be transmitted at the end of the scanned data or not. Default = On.

(9140051.)

*On

(9140050.)

Off

Number System

The numeric system digit of a U.P.C. symbol is normally transmitted at the beginning of the scanned data, but the unit can be programmed so it will not transmit it. To prevent transmission, scan Off. Default = On.

(9140061.)

* On

Addenda

(9140060.)

Off

This selection adds 2 or 5 digits to the end of all scanned UPC-E data.Default = Off for both 2 Digit and 5 Digit Addenda.

(9140071.)

2 Digit Addenda On

(9140070.)

* 2 Digit Addenda Off

(9140081.)

5 Digit Addenda On

(9140080.)

* 5 Digit Addenda Off

UPC-E1

Most U.P.C. barcodes lead with the 0 number system. For these codes, use UPC-E0. If you need to read codes that lead with the 1 number system, use the UPC-E1 On selection. Default = Off.

(9140091.)

UPC-E1 On

(9140090.)

EAN/JAN-13

* UPC-E1 Off

<Default All EAN/JAN Settings>

(915000.)

Default All EAN/JAN-13 Settings

On/Off

(9150011.)

*On

(9150010.)

Off

Note: If you want to convert UPC-A barcodes to EAN-13 format, scan the

UPC-A Off barcode.

Check Digit

This selection allows you to specify whether the check digit should be transmitted at the end of the scanned data or not. Default = On.

(9150021.)

* On

(9150020.)

Off

Addenda

This selection adds 2 or 5 digits to the end of all scanned EAN/JAN-13 data. Default = Off for both 2 Digit and 5 Digit Addenda.

(9150031.)

2 Digit Addenda On

(9150030.)

* 2 Digit Addenda Off

(9150041.)

5 Digit Addenda On

(9150040.)

* 5 Digit Addenda Off

Addenda Required

When Required is scanned, the scanner will only read EAN/JAN-13 barcodes that have addenda. Default = Not Required.

(9150051.)

Required

Addenda Separator

(9150050.)

* Not Required

When this feature is On, there is a space between the data from the barcode and the data from the addenda. When turned Off, there is no space. Default

= On.

(9150061.)

* On

ISBN Translate

(9150060.)

Off

When On is scanned, EAN-13 Bookland symbols are translated into their equivalent ISBN number format. Default = Off.

(9150071.)

On

(9150070.)

* Off

EAN/JAN-8

<Default All EAN/JAN-8 Settings>

On/Off

(916000.)

Default All EAN/JAN-8 Settings

(9160011.)

* On

Check Digit

(9160010.)

Off

This selection allows you to specify whether the check digit should be transmitted at the end of the scanned data or not. Default = On.

(9160021.)

* On

Addenda

(9160020.)

Off

This selection adds 2 or 5 digits to the end of all scanned EAN/JAN-8 data. Default = Off for both 2 Digit and 5 Digit Addenda.

(9160031.)

2 Digit Addenda On

(9160041.)

5 Digit Addenda On

(9160030.)

- 2 Digit Addenda Off

(9160040.)

- 5 Digit Addenda Off

Addenda Required

When Required is scanned, the scanner will only read EAN/JAN-8 barcodes that have addenda. Default = Not Required.

(9160051.)

Required

(9160050.)

* Not Required

Addenda Separator

When this feature is On, there is a space between the data from the barcode and the data from the addenda. When turned Off, there is no space. Default

= On.

(9160061.)

* On

(9160060.)

Off

MSI

On/Off

<Default All MSI Settings>

(917000.)

Default All MSI Settings

(9170011.)

On

(9170010.)

* Off

Check Characte

MSI barcodes use different types of check characters. You can conﬁgure the barcode scanner to read the MSI barcode using the check character. Default = Validate MOD 10, but Don’t Transmit

When Check Character is set to Validate MOD 10 and Transmit, the scanner will only read MSI barcodes printed with the speciﬁed type check character(s), and will transmit the character(s) at the end of the scanned data.

When Check Character is set to Validate MOD 10, but Don’t Transmit, the unit will only read MSI barcodes printed with the speciﬁed type check character(s), but will not transmit the check character(s) with the scanned data.

(9170020.)

* Validate MOD 10, but Don’t Transmit

(9170021.)

Validate MOD 10 and Transmit

(9170026.)

Disable MSI Check Characters

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 4-48. Minimum Default = 4, Maximum Default = 48.

(917004.)

Minimum Message Length

(917003.)

Maximum Message Length

GS1 DataBar Omnidirectional

< Default All GS1 DataBar Omnidirectional Settings >

On/Off

(918000.)

Default All GS1 DataBar Omnidirectional Settings

(9180011.)

* On

GS1 DataBar Limited

(9180010.)

Off

< Default All GS1 DataBar Limited Settings >

On/Off

(919000.)

Default All GS1 DataBar Limited Settings

(9190011.)

* On

(9190010.)

Off

On/Off

(920000.)

Default All GS1 DataBar Expanded Settings

(9200011.)

* On

(9200010.)

Off

Message Length

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 4-74. Minimum Default = 4, Maximum Default = 74.

(920003.)

Minimum Message Length

(920002.)

Maximum Message Length

On/Off

(924000.)

Default All PD417 Settings

(9240011.)

* On

Message Length

(9240010)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-2750. Minimum Default = 1, Maximum Default = 2750.

(924003.)

Minimum Message Length

(924002.)

Maximum Message Length

On/Off

(928000.)

Default All QR Code Settings

This selection applies to both QR Code and Micro QR Code.

(9280011.)

* On

Message Length

(9280010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-7089. Minimum Default = 1, Maximum Default = 7089.

(928003.)

Minimum Message Length

(928002.)

Maximum Message Length

On/Off

(930000.)

Default All Data Matrix Settings

(9300011.)

* On

Message Length	(9300010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-3116. Minimum Default = 1, Maximum Default = 3116.

(930002.)

Minimum Message Length

(930003.)

Maximum Message Length

On/Off

(931000.)

Default All Aztec Code Settings

(9310011.)

* On

Message Length

(9310010.)

off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-3832. Minimum Default = 1, Maximum Default = 3832.

(931003.)

Minimum Message Length

(931002.)

Maximum Message Length

(936000.)

Default All China Post (Hong Kong 2 of 5)Settings

On/Off

(9360011.)

On

Message Length

(9360010.)

* Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 2-80. Minimum Default = 4, Maximum Default = 80.

(936003.)

Minimum Message Length

(936002.)

Maximum Message Length

Korea Post

On/Off

<Default All Korea Post Settings>

(937000.)

Default All Korea Post Settings

(9370011.)

On

Message Length

(9370010.)

* Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 2-80. Minimum Default = 4, Maximum Default = 48.

(937003.)

Minimum Message Length

(937002.)

Maximum Message Length

Check Digit

This selection allows you to specify whether the check digit should be transmitted at the end of the scanned data. Default = Don’t Transmit.

(9370041.)

On

Han Xin Code

On/Off

(9370040.)

* Off

<Default All Han Xin Code Settings>

(932000.)

Default All Han Xin Code Settings

(9320011.)

On

Message Length

(9320010.)

* Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-1000. Minimum Default = 1, Maximum Default = 1000.

(932003.)

Minimum Message Length

(932002.)

Maximum Message Length

Maxi code

<defalt all maxi code settings>

On/Off

(9290011.)

On

(929000.)

Message Length

(9290010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-150. Minimum Default = 1, Maximum Default = 150.

(929003.)

Minimum Message Length

(929002.)

Maximum Message Length

Micropdf

<defalt all micropdf settings>

On/Off

(925000.)

(9250011.)

On

Message Length

(9250010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-366. Minimum Default = 1, Maximum Default = 366.

(925003.)

Minimum Message Length

(925002.)

Maximum Message Length

Composites

<defalt all composites settings>

On/Off

(926000.)

(9260011.)

On

Message Length

(9260010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-2435. Minimum Default = 1, Maximum Default = 2435.

(926004.)

Minimum Message Length

(926003.)

Maximum Message Length

Codablock A

<defalt all composites settings>

On/Off

(922000.)

(9220011.)

On

Message Length

(9220010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-600. Minimum Default = 1, Maximum Default = 600.

(922003.)

Minimum Message Length

(922002.)

Maximum Message Length

On/Off

(923000.)

(9230011.)

On

Message Length

(9230010.)

Off

Scan the barcodes below to change the message length. Refer to Message Length Description for additional information. Minimum and Maximum lengths = 1-2048. Minimum Default = 1, Maximum Default = 2048.

(923003.)

Minimum Message Length

(923002.)

Maximum Message Length

Disable GS1 Data Matrix

Scan the barcode to disable GS1 Data Matrix

On

Off

Show Software Revision

Scan the barcode below to output the current software revision, unit serial number, and other product information.

(809004?.)

Show Revision

Problem: The barcode scanner does not work.

possible reason：

- The barcode scanner is not powered, check the power of the equipment.

- If you are using an incorrect cable, use the cable that was originally conﬁgured.

- The cable interface is loose and reconnected.

- Check if the button is normal.

Problem: The barcode scanner scans normally, but the data output is incorrect. possible reason：

- The cable interface is loose and reconnected.

- Barcode scanner may not be conﬁgured to display the correct terminal.

- If you are using a USB to RS232 cable, if the data output is garbled, it may be that the data reception speed of the device does not match the output speed of the barcode scanner.

Problem: Barcode scanner does not decode some barcodes. possible reason：

- The barcode is defective. Try to scan the same type of test barcode to see if it can be interpreted.

- The distance between the barcode scanner and the barcode is not suitable. Please move closer or move away the barcode.

- For barcodes with poor print quality, the preferred reading distance is 5-10 cm.

- Conﬁrm that your device is enabled for this barcode type.

Problem: Other conditions cannot be decoded. possible reason：

- Turn off the device power; properly connect the device to the barcode scanner; turn on the device and test it.

- If the problem still cannot be solved, please contact the dealer or the manufacturer.

M aintenance

- Stains and dust on the scanning window can sometimes affect the normal operation of the barcode scanner. When cleaning, use a good quality tissue to wipe gently, or use a soft cloth to clean.

If you use a paper with poor paper quality for a long time, it will damage the surface ﬁnish of the window and affect the reading effect of the barcode scanner.

- The outer shell of the barcode scanner can be wiped with a soft, clean cloth. If necessary, add a small amount of detergent to the water, wipe it with a soft cloth and rub it.

- Do not spray any liquid on the window.

- The scanning window must be kept clean and the supplier is not liable for damage caused by improper maintenance.

Customer Service

If you need help installing or troubleshooting a device, please contact us

| Symbology | AIM |  |  |  |
| --- | --- | --- | --- | --- |
|  | ID | Possible Modiﬁers （m） | ID | Hex |
| All Symbologies |  |  |  | 99 |
| Codabar | ]Fm | 0-1 | a | 61 |
| Code 11 | ]H3 |  | h | 68 |
| Code 128 | ]Cm | 0, 1, 2, 4 | j | 6A |
| Code 32 Pharmaceutical (PARAF) | ]X0 |  | < | 3C |
| Code 39 (supports Full ASCII mode) | ]Am | 0, 1, 3, 4, 5,7 | b | 62 |
| TCIF Linked Code 39 (TLC39) | ]L2 |  | T | 54 |
| Code 93 and 93i | ]Gm | 0-9, A-Z, a-m | i | 69 |
| EAN | ]Em | 0, 1, 3, 4 | d | 64 |
| EAN-13 (including Bookland EAN) | ]E0 |  | d | 64 |
| EAN-13 with Add-On | ]E3 |  | d | 64 |
| EAN-13 with Extended Coupon | ]E3 |  | d | 64 |
| EAN-8 | ]E4 |  | D | 44 |
| EAN-8 with Add-On | ]E3 |  | D | 44 |

Reference Charts Symbology Charts Linear Symbologies

| Symbology | AIM |  |  |  |
| --- | --- | --- | --- | --- |
|  | ID | Possible Modiﬁers （m） | ID | Hex |
| GS1 |  |  |  |  |
| GS1 DataBar | ]em | 0 | y | 79 |
| GS1 DataBar Limited | ]em |  | { | 7B |
| GS1 DataBar Expanded | ]em |  | } | 7D |
| GS1-128 | ]C1 |  | I | 49 |
| 2 of 5 |  |  |  |  |
| China Post (Hong Kong 2 of | ]X0 |  | Q | 51 |
| Interleaved 2 of 5 | ]Im | 0, 1, 3 | e | 65 |
| Matrix 2 of 5 | ]X0 |  | m | 6D |
| NEC 2 of 5 | ]X0 |  | Y | 59 |
| Straight 2 of 5 IATA | ]Rm | 0, 1, 3 | f | 66 |
| Straight 2 of 5 Industrial | ]S0 |  | f | 66 |
| MSI | ]Mm | 0, 1 | g | 67 |
| Telepen | ]Bm |  | t | 74 |
| UPC |  | 0, 1, 2, 3, 8, |  |  |
| UPC-A | ]E0 |  | c | 63 |
| UPC-A with Add-On | ]E3 |  | c | 63 |
| UPC-A with Extended Coupon | ]E3 |  | c | 63 |
| UPC-E | ]E0 |  | E | 45 |
| UPC-E with Add-On | ]E3 |  | E | 45 |
| UPC-E1 | ]X0 |  | E | 45 |
| Add Code ID |  |  |  | 5C 80 |
| Add AIM Code ID |  |  |  | 5C 81 |
| Add Backslash |  |  |  | 5C 5C |
| Batch Mode Quantilty |  |  | 5 | 35 |

5)

9, A, B, C

6

2D Symbologies

| Symbology | AIM |  |  |  |
| --- | --- | --- | --- | --- |
|  | ID | Possible Modiﬁers （m） | ID | Hex |
| All Symbologies |  |  |  | 99 |
| Aztec Code | ]zm | 0-9, A-C | z | 7A |
| Chinese Sensible Code (Han Xin Code) | ]X0 |  | H | 48 |
| Codablock A | ]O6 | 0, 1, 4, 5, | V | 56 |
| Codablock F | ]Om | 0, 1,64, 5, | q | 71 |
| Code 49 | ]Tm | 0, 1, 2, 4 | l | 6C |
| Data Matrix | ]dm | 0-6 | w | 77 |
| GS1 | ]em | 0-3 | y | 79 |
| GS1 Composite | ]em | 0-3 | y | 79 |
| GS1 DataBar Omnidirecti | ]em | 0-3 | y | 79 |
| MaxiCode | ]Um | 0-3 | x | 78 |
| PDF417 | ]Lm | 0-2 | r | 72 |
| MicroPDF417 | ]Lm | 0-5 | R | 52 |
| QR Code | ]Qm | 0-6 | s | 73 |
| Micro QR Code | ]Qm |  | s | 73 |

Post

Postal Symbologies

| Symbology | AIM |  |  |  |
| --- | --- | --- | --- | --- |
|  | ID | Possible Modiﬁers （m） | ID | Hex |
| All Symbologies |  |  |  | 99 |
| Australian Post | ]X0 |  | A | 41 |
| British Post | ]X0 |  | B | 42 |
| Canadian Post | ]X0 |  | C | 43 |

| Symbology | AIM |  |  |  |
| --- | --- | --- | --- | --- |
|  | ID | Possible Modiﬁer s | ID | Hex |
| China Post | ]X0 | （m） | Q | 51 |
| InfoMail | ]X0 |  | , | 2c |
| Intelligent Mail Bar Code | ]X0 |  | M | 4D |
| Japanese Post | ]X0 |  | J | 4A |
| KIX (Netherlands) | ]X0 |  | K | 4B |
| Korea Post | ]X0 |  | ? | 3F |
| Planet Code | ]X0 |  | L | 4C |
| Postal-4i | ]X0 |  | N | 4E |
| Postnet | ]X0 |  | P | 50 |

ASCII Conversion Chart

| Hex | Dec | Char |
| --- | --- | --- |
| 00 | 0 | NUL （Null char.） |
| 01 | 1 | SOH （Start of Header） |
| 02 | 2 | STX （Start of Text） |
| 03 | 3 | ETX （End of Text） |
| 04 | 4 | EOT （End of Transmission) |
| 05 | 5 | ENQ （Enquiry） |
| 06 | 6 | ACK （Acknowledgment） |
| 07 | 7 | BEL （Bell） |
| 08 | 8 | BS （Backspace） |
| 09 | 9 | HT （Horizontal Tab） |
| 0a | 10 | LF （Line Feed） |
| 0b | 11 | VT （Vertical Tab） |
| 0c | 12 | FF （Form Feed） |
| 0d | 13 | CR （Carriage Return） |
| 0e | 14 | SO （Shift Out） |
| 0f | 15 | SI （Shift In） |
| 10 | 16 | DLE （Data Link Escape） |
| 11 | 17 | DC1 （XON）（Device Control 1） |
| 12 | 18 | DC2 （Device Control 2） |
| 13 | 19 | DC3（XOFF）（Device Control 3） |
| 14 | 20 | DC4 （Device Control 4） |
| 15 | 21 | NAK （Negative Acknowledgment） |
| 16 | 22 | SYN （Synchronous Idle） |
| 17 | 23 | ETB （End of Trans. Block） |
| 18 | 24 | CAN （Cancel） |
| 19 | 25 | EM （End of Medium） |
| 1a | 26 | SUB （Substitute） |
| 1b | 27 | ESC （Escape） |
| 1c | 28 | FS （File Separator） |
| 1d | 29 | GS （Group Separator） |
| 1e | 30 | RS （Request to Send） |
| 1f | 31 | US （Unit Separator） |
| 20 | 32 | SP （Space） |
| 21 | 33 | ! （Exclamation Mark） |
| 22 | 34 | " （Double Quote） |
| 23 | 35 | # （Number Sign） |
| 24 | 36 | $ （Dollar Sign） |

|  | 25 | 37 | % （Percent） |  |
| --- | --- | --- | --- | --- |
|  | 26 | 38 | & （Ampersand） |  |
|  | 27 | 39 | ` （Single Quote） |  |
|  | 28 | 40 | (（Right / Closing Parenthesis） |  |
|  | 29 | 41 | )（Right / Closing Parenthesis） |  |
|  | 2a | 42 | * （Asterisk） |  |
|  | 2b | 43 | + （Plus） |  |
|  | 2c | 44 | ,（Comma） |  |
|  | 2d | 45 | -（Minus / Dash） |  |
|  | 2e | 46 | . （Dot） |  |
|  | 2f | 47 | / （Forward Slash） |  |
|  | 30 | 48 | 0 |  |
|  | 31 | 49 | 1 |  |
|  | 32 | 50 | 2 |  |
|  | 33 | 51 | 3 |  |
|  | 34 | 52 | 4 |  |
|  | 35 | 53 | 5 |  |
|  | 36 | 54 | 6 |  |
|  | 37 | 55 | 7 |  |
|  | 38 | 56 | 8 |  |
|  | 39 | 57 | 9 |  |
|  | 3a | 58 | ：（Colon） |  |
|  | 3b | 59 | ；（Semi-colon） |  |
|  | 3c | 60 | < （Less Than） |  |
|  | 3d | 61 | = （Equal Sign） |  |
|  | 3e | 62 | > （Greater Than） |  |
|  | 3f | 63 | ? （Question Mark） |  |
|  | 40 | 64 | @ （AT Symbol） |  |
|  | 41 | 65 | A |  |
|  | 42 | 66 | B |  |
|  | 43 | 67 | C |  |
|  | 44 | 68 | D |  |
|  | 45 | 69 | E |  |
|  | 46 | 70 | F |  |
|  | 47 | 71 | G |  |
|  | 48 | 72 | H |  |
|  | 49 | 73 | I |  |
|  | 4a | 74 | J |  |
|  | 4b | 75 | K |  |
|  | 4c | 76 | L |  |
|  | 4d | 77 | M |  |
|  | 4e | 78 | N |  |
|  | 4f | 79 | O |  |

|  | 50 | 80 | P |  |
| --- | --- | --- | --- | --- |
|  | 51 | 81 | Q |  |
|  | 52 | 82 | R |  |
|  | 53 | 83 | S |  |
|  | 54 | 84 | T |  |
|  | 55 | 85 | U |  |
|  | 56 | 86 | V |  |
|  | 57 | 87 | W |  |
|  | 58 | 88 | X |  |
|  | 59 | 89 | Y |  |
|  | 5a | 90 | Z |  |
|  | 5b | 91 | [ （Left / Opening Bracket） |  |
|  | 5c | 92 | \（Back Slash） |  |
|  | 5d | 93 | ] （Right / Closing Bracket） |  |
|  | 5e | 94 | ^ （Caret / Circumﬂex） |  |
|  | 5f | 95 | _ （Underscore） |  |
|  | 60 | 96 | ' （Grave Accent） |  |
|  | 61 | 97 | a |  |
|  | 62 | 98 | b |  |
|  | 63 | 99 | c |  |
|  | 64 | 100 | d |  |
|  | 65 | 101 | e |  |
|  | 66 | 102 | f |  |
|  | 67 | 103 | g |  |
|  | 68 | 104 | h |  |
|  | 69 | 105 | i |  |
|  | 6a | 106 | j |  |
|  | 6b | 107 | k |  |
|  | 6c | 108 | l |  |
|  | 6d | 109 | m |  |
|  | 6e | 110 | n |  |
|  | 6f | 111 | o |  |
|  | 70 | 112 | p |  |
|  | 71 | 113 | q |  |
|  | 72 | 114 | r |  |
|  | 73 | 115 | s |  |
|  | 74 | 116 | t |  |
|  | 75 | 117 | u |  |
|  | 76 | 118 | v |  |
|  | 77 | 119 | w |  |
|  | 78 | 120 | x |  |
|  | 79 | 121 | y |  |
|  | 7a | 122 | z |  |
|  | 7b | 123 | { （Left/ Opening Brace） |  |

|  | 7c | 124 | ¦ （Vertical Bar） |  |
| --- | --- | --- | --- | --- |
|  | 7d | 125 | } （Right/Closing Brace） |  |
|  | 7e | 126 | ~ （Tilde） |  |
|  | 7f | 127 | DEL （Delete） |  |

Sample Symbols

UPC-A

01234567890	Interleaved 2 of 5

Code 128

Straight 2 of 5Industrial

123456

Matrix 2 of 5

6543210

Code 93

123456-9$	Straight 2 of 5Industrial

Matrix 2 of 5

123456

6543210	GS1DataBar

PDF417

Codabar

Data Matrix

QRCode

Numbers

TestSymbol

Aztec

MaxiCode

Test Message

Micro PDF417

Test Message

Programming Charts

(K0K.)

0

(K2K.)

2

(K4K.)

4

(K6K.)

6

(K1K.)

1

(K3K.)

3

(K5K.)

5

(K7K.)

7

(K8K.)

8

(KAK.)

A

(KCK.)

C

(KEK.)

E

(K9K.)

(KBK.)

B

(KDK.)

D

(KFK.)

F

(800002.)

Save

(800000.)

Discard

Note: If an error occurs while scanning a letter or number (before scanning the "Save" barcode), scan the "Discard" barcode, rescan the correct letter or number, and then scan the "Save" barcode.